COMBAT (CCP4: Supported Program)

NAME

combat - produces an MTZ file in multirecord form suitable for input

to SCALA.

SYNOPSIS

combat hklin foo.mtz hklout foo.mtz

[Keyworded input]

DESCRIPTION

COnvert to Multi BATch MTZ files. This program converts output from

assorted data processing programs into a multirecord MTZ file suitable

for input to the CCP4 scaling and merging programs (SCALA and

TRUNCATE). Only mosflm writes such an MTZ file. The program is a

replacement program for ROTAPREP which was replaced as it was very

difficult to support.

Input types accepted at present are: DATRED_RIGAKU, DENZO, JIMS, MTZF,

MTZI, MUF, MUI, RAXISDUMP, SAINT, various SCALEPACK options , TEXHKL,

TEXSAN, USER, WEISS, XDSFULL, XDSUNIQUE.

Some of these are quite rare, but it seems sensible to keep them. It

would be trivial to extend the program for any format.

Warning: It is quite likely that the authors of processing programs

will change their formats without telling CCP4. If you have any

queries or knowledge of such changes please contact us.

Datasets and Harvesting

Datasets should be assigned project and dataset names for harvesting

(and some other functionality), and these should be assigned in COMBAT

using the PNAME and DNAME keywords. These will be written to the the

output MTZ file, and be inherited by subsequent programs.

INPUT FORMATS

The following arguments to the INPUT keyword are supported. The most

commonly used arguments are those used to convert existing MTZ files,

or options for DENZO/SCALEPACK outputs (keywords SCALEPACK, SCAL_NM

and SCAL_NM2). These are described first. The items in the

descriptions of the format have the following interpretations:

h k l

Miller indices (possibly unique ones);

F sigF

Measured structure factor amplitude and its standard deviation;

I sigI

Measured intensity and its standard deviation;

delI sigdelI

Anomalous intensity difference and its standard deviation;

ident / ibatch

Batch number or equivalent;

mpart / fpart

Partiality flag (in some convention).

MTZI

Standard MTZ file containing h k l I sigI or h k l I+ sigI+ I- sigI-.

It is possible to read in a standard MTZ format file and produce a

`multirecord' MTZ file. You should only use merged data as a reference

`batch' for SCALA.

MTZF

Standard MTZ file containing either h k l F sigF or h k l F(+) sigF(+)

F(-) sigF(-). It is possible to read such an MTZ format file and

produce a `multirecord' MTZ file. You should only use merged data as a

reference `batch' for SCALA. This is probably most useful for

producing test data from Fcalc

SCALEPACK

Output from SCALEPACK [h k l I SigI or h k l I+ SigI+ I- SigI-] in

format (3i4,4f8.0).

N.B. Run SCALEPACK2MTZ to generate an MTZ file containing h k l I sigI

or h k l I+ sigI+ I- sigI- then input this as MTZI.

SCAL_NM2

Output from SCALEPACK option NOMERGE original indices [h k l h0 k0 l0

ibatch I SigI] in format (6I4,i6,7x,2f8.0). Scales have been applied

and partials summed. The Friedel pairs are separate in this case.

SCAL_NM

Output from SCALEPACK option NOMERGE no partials [h k l ibatch isym I

SigI (Fpart)] in format (4i4,i3,2f8.0,f7.2). Scales have been applied

and partials summed. The Friedel pairs are separate in this case.

DATRED_RIGAKU

[h k l F sigF] in format (3i4,4f8.1).

JIMS

MADNES for050 files (but using ABSURD is strongly preferred).

[h k l I sigI] in format (3i5,2f10.2).

MUI

For intensities output from the XENTRONICS and XENGEN data processing

packages.

[format (3i4, 1x, F6.4, 6(1x,f8.2), 1x, i2).]

MUF

For amplitudes output from the XENTRONICS and XENGEN data processing

packages.

[format (3i4, 1x, F6.4, 6(1x,f8.2), 1x, i2).]

RAXISDUMP

Output from Raxis READBF of mergefile.

[mpart h k l I sigI ibatch] in format (21x, I4, 4x, 3i4, 8x, 2f10.3,

i4).

Note: if you use the `log' file produced under unix rather than

the `output' file you may need to change the format to eat an

extra space, i.e. start it with `22x'. Note also that you may

find alternative processing packages to be better than the

Raxis software.

DENZO

This option may cause serious trouble further down the line. Do not

use unless you know exactly what you are doing.

Concatenated output files from DENZO. These contain HEADER blocks and

reflection data. [h k l fsqp I sigI fpart phi] in format (3I4, 2X,

2F8.0, 7X, F6.0, 6X, 2F7.0, 14X, 2F6.0)

Care is needed when doing this, since no post-refinement has been

done. We now prefer to run SCALEPACK to refine the cell and

orientation, then either run DENZO again to re-integrate the images

without re-refining the cell etc. (however there may still be problems

with partiality flags), or output an unmerged file from scalepack (see

keywords SCAL_NM and SCAL_NM2).

N.B. If using the commercialised Denzo identified like:

HKL data processing system Version 1.0 FT1.31 Rev1.4

you need to use the Denzo command

film york output file <filename>

to produce a suitable format for `combat'. The `york' output

can also be read into `scalepack' with

format denzo_york1

SAINT

Output from SAINT, one batch only (not fully tested).

TEXSAN

Output from TEXSAN data processing package for Rigaku diffractometer.

[h k l ident I sigI] in format (3i4, i3, 5x, f8.1, f7.1).

TEXHKL

Output from TEXSAN data processing package for Rigaku diffractometer.

[h k l I sigI scale] in format (3x, 3i4, 2F10.2, 29x, F7.4).

USER

ASCII data file in a user-defined format, see LABEL and FORMAT

keywords.

WEISS

Output from the WEISS data processing package for Japanese Weissenberg

data. [h k l mpart I sigI ident] in format (5x, 4i4, 2f8.2, i10).

XDSUNIQUE

Output from Kabsch's XDS data processing package (or the MAR Research

version) in the standard text file UNIQUE.HKL from the CORRECT stage.

[The output of the MARSCALE program (and possibly the original XSCALE)

can also be put in this form, although there is probably no reason to

prefer these to scala]. This file contains symmetry averaged

reflection intensities and anomalous differences after scaling.

[h k l I sigI delI sigdelI] in format (3I5, 4E12.4)

Since UNIQUE output is scaled and merged, it is sensible to use

`f2mtz' to produce an MTZ file for direct input to `truncate'.

XDSFULL

Output from Kabsch's XDS data processing package (or the MAR Research

version) in the standard direct access binary file XDS.HKL from the

CORRECT stage. This contains scaled intensities of all reflections

before symmetry averaging. Note that this file type will not be

generally portable between computer systems - run `combat' on the

system which generated xds.hkl.

DESCRIPTION OF THE OUTPUT MTZ FILE

The output file has the column labels:

H K L

Miller indices.

M/ISYM

256*M + ISYM, where the partiality flag M=0 for fully recorded

reflexions and M=1 for partials, and ISYM is the symmetry

number.

BATCH

Batch (serial) number for this film/image.

I, sigI

Corrected integrated intensity and its standard deviation.

FRACTIONCAL

This is Calculated fraction recorded*1000 (>500 if partial more

than half present). For most inputs FRACTIONCAL is a dummy set

to 3000 or 501 for a partial.

RATDELSD

1000*ratio of worst delta(I)/STD.DEVN found within a film pack.

IPR, sigIPR

is meant to be corrected profile-fitted intensity and SD. Here

IPR and sigIPR are set equal to I and sigI if there is no

appropriate information input.

XDET, YDET, ROT

Detector position and PHI angle. This information is available

for DENZO & SAINT only.

KEYWORDED INPUT

Available keywords are:

ADDBATCH, BATCH, CELL, DETECTOR, DNAME, WAVELENGTH,END, FORMAT,

INPUT, LABEL, LABIN, MONITOR, PHIRANGE, PNAME, REINDEX, REJECT,

RESOLUTION, SCALE, SYMMETRY, TITLE

Compulsory input keywords for some options:

INPUT <type>

The allowed <type>s are those listed above.

BATCH <batch number>

This specifies the `batch' number for data in the multirecord MTZ

output. (BATCH is not required for DENZO, SCAL_NM, SCAL_NM2, SAINT or

USER input if a batch column is present in the input file. It is not

required for WEISS or TEXSAN inputs but if given it will override the

IDENT value.)

CELL <a> <b> <c> [ <alpha> <beta> <gamma> ]

Cell dimensions in Angstroems and angles in degrees; the angles

default to 90 degrees. This keyword is not compulsory for MTZF, MTZI,

SCAL_NM or DENZO input, but if given it will override the cell

dimensions from the file headers. This keyword is compulsory for other

input types.

LABIN <program label> = <file label> ...

Only compulsory for MTZF or MTZI input.

Column assignments for conventional labels F and SIGF are required for

MTZF input, or for I and SIGI for MTZI input.

LABEL <file label 1> <file label 2> ...

Column assignments for user-defined input format (INPUT USER). <file

label n> must be chosen from H, K, L, M, BATCH, I, SIGI (or F, SIGF,

or F(+) SIGF(+) F(-) SIGF(-)), IPR, SIGIPR, FRACTION and given in the

order in which they occur in the input file. The indices H, K, L are

compulsory. M is optional and defaults to 0, i.e. fully recorded

reflexions. The batch number should either be read in from a BATCH

column or taken from the BATCH keyword (the former takes precedence if

both are present). Either I, SIGI or F, SIGF must be given; if the

latter, then F is squared and I/SIGI output. The remaining columns

IPR, SIGIPR, FRACTION are optional.

SYMMETRY <SG number> | <SG symbol>

Specifies the spacegroup for the data. Compulsory if not MTZF or MTZI

input.

Optional input keywords:

For input types which contain batch number information the following

two keywords determine the transformation of the input numbers. Note

that the "MISBATCH" command is no longer needed to list missing

batches and has been removed.

ADDBATCH <offset>

<offset> is added to all input batch numbers (default 0).

PHIRANGE <divisions>

(DENZO only, default 1.) Each batch is split into a number <divisions>

of divisions for scaling. The output batch number will be:

<divisions>*(batch - 1) + phi_step_counter + <offset>

FORMAT <format string>

Supply a format specifier for user-defined input (see INPUT USER). The

argument must be a valid FORTRAN fixed format string, such as might be

given in a FORMAT statement, including the brackets and quoted. E.g.:

FORMAT '(6(6X,F8.3))'

will read records comprising six numbers each preceded by a

six-character-wide field which will be skipped. It is not possible to

read more than one reflexion from each input line.

The format should be reading real numbers (F), and not integers (I),

so if your input file contains integers, then they should be read as

reals with e.g. F6.0 rather than I6. If the supplied FORMAT statement

contains I descriptors then COMBAT will automatically convert them to

the correct F format for you.

Make use of the X descriptor to skip unwanted columns. Under Unix, the

cut (1) command may be useful for reformatting the input columns if,

for instance, the relevant fields aren't in fixed positions. If the

FORMAT keyword is not given for user-defined input, then the default

format specifier '*' is used.

PNAME <project name>

[default: 'unknown']

This keyword can be used to assign a project name for HKLOUT. For

input options MTZF and MTZI, a project name given by this keyword

overrides any in HKLIN.

This keyword is not compulsory, but is advisable to obtain a sensible

name.

A dataset, as listed in the MTZ header, is specified by a

project-name/dataset-name pair. The project-name specifies a

particular structure solution project, while the dataset-name

specifies a particular dataset contributing to the structure solution.

An entry in the PNAME keyword should therefore be accompanied by a

corresponding entry in the DNAME keyword.

DNAME <dataset name>

[default: 'unknownddmmyy']

This keyword can be used to assign a dataset name for HKLOUT. For

input options MTZF and MTZI, a dataset name given by this keyword

overrides any in HKLIN.

This keyword is not compulsory, but is advisable to obtain a sensible

name rather than 'unknownddmmyy' (where ddmmyy is the date, with no

spaces).

A dataset, as listed in the MTZ header, is specified by a

project-name/dataset-name pair. The project-name specifies a

particular structure solution project, while the dataset-name

specifies a particular dataset contributing to the structure solution.

An entry in the DNAME keyword should therefore be accompanied by a

corresponding entry in the PNAME keyword.

WAVELENGTH <wavelength>

Assign wavelength to dataset.

MONITOR <nmon>

Every <nmon>-th reflection is monitored (printed out). Default: no

monitoring.

REJECT <hrej> <krej> <lrej>

The arguments specify indices of reflections to be excluded from the

output file (these indices should lie in the spacegroup's chosen

asymmetric unit). All reflections equivalent to this set of indices

will be rejected. This is a desperate measure to weed out bad

reflections (usually overloads) without repeating the data processing.

This line can be repeated up to 200 times.

REINDEX <transformation>

<transformation> specifies how to transform the original indices for

the output. It is specified in the form k,h,-l; h,-k,-h/2-1/2; etc.

The default is no transformation.

RESOLUTION <resmin> [ <resmax> ]

Rejects reflections outside the given resolution range in �s or

4s**2/lambda**2.

SCALE <scale>

The input intensities are multiplied by <scale> (default 1.0).

TITLE <title>

<title> is written to the output file.

DETECTOR <xmin> <xmax> <ymin> <ymax>

limits of detector coordinates XDET, YDET. These are needed for the

detector scaling options in Scala.

END

to finish.

SEE ALSO

scala, f2mtz, Data Harvesting, CAD, `The marxds marscale manual', MAR

Research.

EXAMPLES

combat HKLIN raxis.dump HKLOUT junk.mtz << +

CELL 63 63 264 90 90 90

MONITOR 1000

BATCH 1                # first batch number

INPUT RAXISDUMP

RESOLUTION 0 0.137

SYMMETRY 89

TITLE   Test combat

PNAME myproject

DNAME native

END

+

combat                       \

HKLIN ESRF/pcrb/pcrb5_e1.sca \

HKLOUT $SCRATCH/pcrb5_e1.mtz    \

<< 'END-mtzrwrot'

MONI 10000

INPUT SCAL_NM2

ADDBATCH 0

CELL 92.004  92.004 175.229  90.000  90.000  90.000

SYMM P41212

TITLE Test on Scalepack - NoMerge- Original Indices L1

PNAME myproject

DNAME native

END

'END-mtzrwrot'

#

AUTHOR

Alun Ashton, CCP4 Daresbury Laboratory.

Origional program was ROTAPREP by Eleanor Dodson, University of York